Neeti Kurulkar

SE Div: A Batch: S2

Roll No: 21038

Write X86/64 ALP to perform multiplication of two 8-bit hexadecimal numbers. Use

successive addition and add and shift method. (use of 64-bit registers is expected).

section .data

msg1 db "Enter Multiplicand : "

len1 equ $ - msg1

msg2 db "Enter Multiplier : "

len2 equ $ - msg2

menu db 10,"1)Successive Addition."

db 10,"2)Add & Shift."

db 10,"3)Exit."

db 10,"Enter Your Choice : "

menuLen equ $ - menu

emsg db "Invalid Choice"

emsgLen equ $ - emsg

resultH dw 0000000000000000h

newline db 10

msg3 db 10,"Result :"

len3 equ $ - msg3

section .bss

multiplicandH resb 3

multiplicandA resb 1

multiplierH resb 3

multiplierA resb 1

choice resb 2

avar resb 4

var resb 2

count resb 1

resultA resb 4

mulcopy resb 1

%macro print 2

mov rax, 1

mov rdi, 1

mov rsi, %1

mov rdx, %2

syscall

%endmacro

%macro read 2

mov rax, 0

mov rdi, 0

mov rsi, %1

mov rdx, %2

syscall

%endmacro

section .text

global _start

_start:

up:

print msg1, len1

read multiplicandA, 3

print msg2, len2

read multiplierA, 3

mov ax, word[multiplicandA]

mov word[avar], ax

call packnum1

mov al, byte[var]

mov byte[multiplicandH], al

mov ax, word[multiplierA]

mov word[avar], ax

call packnum1

mov al, byte[var]

mov byte[multiplierH], al

l1:

print menu, menuLen

read choice, 2

cmp byte[choice], 31h

je case1

cmp byte[choice], 32h

je case2

cmp byte[choice], 33h

je exit

print emsg, emsgLen

jmp l1

case1:

mov al, byte[multiplierH]

mov byte[count], al

bac:

mov edx, 0h

mov dl, byte[multiplicandH]

add word[resultH], dx

dec byte[count]

jnz bac

jmp printresult

case2:

mov al, byte[multiplierH]

mov byte[mulcopy], al

mov byte[count], 8

back1:

shl word[resultH], 01

shl byte[mulcopy], 01

jnc s1

xor ax, ax

mov al, byte[multiplicandH]

add word[resultH], ax

s1:

dec byte[count]

jnz back1

printresult:

print msg3, len3

mov ax, word[resultH]

mov word[var], ax

call asciiConv

mov eax, dword[avar]

mov dword[resultA], eax

print resultA, 4

print newline, 1

exit:

mov rax, 60

mov rdi, 00

syscall

packnum1:

mov rsi, avar

mov ebx, 0

mov byte[count], 2

mov eax, 0

back11:

mov al, [rsi]

rol bl, 4

cmp al, 39h

jbe sub30

sub al, 07h

sub30:

sub al, 30h

or bl, al

inc rsi

dec byte[count]

jnz back11

mov byte[var],bl

ret

asciiConv:

mov r8, avar

mov byte[count], 4

mov ax, [var]

back:

rol ax, 04h

mov bl, al

and bl, 0Fh

cmp bl, 09h

jbe add30

add bl, 07h

add30:

add bl, 30h

mov [r8], bl

inc r8

dec byte[count]

jnz back

ret

Output: