Neeti Kurulkar

SE Div: A Batch: S2

Roll No: 21038

Write X86/64 ALP to detect protected mode and display the values of GDTR, LDTR, IDTR, TR and MSW Registers.

section .data

proMsg db 10,10,"The contents of the system registers are as follows : "

proMsgLen equ $ - proMsg

gdtrMsg db 10,10,"Contents of GDTR : ",10

gdtrMsgLen equ $ - gdtrMsg

ldtrMsg db 10,10,"Contents of LDTR : ",10

ldtrMsgLen equ $ - ldtrMsg

idtrMsg db 10,10,"Contents of IDTR : ",10

idtrMsgLen equ $ - idtrMsg

trMsg db 10,10,"Contents of TR : ",10

trMsgLen equ $ - trMsg

mswMsg db 10,10,"Contents of MSW (CR0) register : ",10

mswMsgLen equ $ - mswMsg

protectedMsg db 10,10,"The Microprocessor is in the protected mode"

protectedMsgLen equ $ - protectedMsg

realMsg db 10,10,"The Microprocessor is in the real mode"

realMsgLen equ $ - realMsg

colon db "  :  "

colonLen equ $ - colon

section .bss

gdtr resd 1     ;store 48-bit GDTR value- (resd(32-bit) + resw(16-bit))

resw 1

ldtr resw 1     ;store 16-bit LDTR

idtr resd 1     ;store 48-bit LDTR value- (resd(32-bit) + resw(16-bit))

resw 1

tr resw 1

msw resd 1

result resb 4

%macro write 2

mov rax,1

mov rdi,1

mov rsi,%1

mov rdx,%2

syscall

%endmacro

section .text

global _start

_start:

smsw eax      ; store the contents of the MSW register into the EAX register.

bt eax, 0     ; copy the 0th least significant bit from EAX to the carry flag.

jc protected_mode	;if CF is set --> protected mode

write realMsg,realMsgLen

jmp endOfProgram

protected_mode :

write protectedMsg , protectedMsgLen

write proMsg , proMsgLen

sgdt [gdtr]   ;store the content of GDTR

sldt [ldtr]   ;store contents of LDTR

str [tr]      ;store conents of tr

smsw [msw]    ;store contents of msw

write gdtrMsg,gdtrMsgLen

mov bx,[gdtr+4]  ;move upper half of gdtr in bx reg.

call disp

mov bx,[gdtr+2]

call disp

write colon , colonLen

mov bx,[gdtr]

call disp

write ldtrMsg,ldtrMsgLen

mov bx,[ldtr]

call disp

write idtrMsg,idtrMsgLen

mov bx,[idtr+4]

call disp

mov bx,[idtr+2]

call disp

write colon , colonLen

mov bx,[idtr]

call disp

write trMsg,trMsgLen

mov bx,[tr]

call disp

write mswMsg,mswMsgLen

mov bx,[msw+2]   ;upper half of CR0 or MSW

call disp

mov bx,[msw]

call disp

endOfProgram:

mov rax, 60

mov rdi, 0

syscall

disp:

mov rdi, result     ;point rdi to result variable

mov cx,04           ;load count of rotation in cl

up1:

rol bl,04           ;rotate number left by four bits

mov dl,bl           ;move lower byte in dl

and dl,0fh          ; get only LSB

cmp dl,09h          ;compare with 39h

jg add_37           ;if greater than 39h skip add 37

add dl,30h

jmp skip1           ;else add 30

add_37: add dl,37h

skip1:  mov [rdi],dl        ;store ascii code in result variable

inc rdi             ;point to next byte

dec cx              ;decrement the count of digits to display

jnz up1             ;if not zero jump to repeat

write result , 4

ret

Output: