Neeti Kurulkar

SE Div: A Batch: S2

Roll No: 21038

Write X86/64 ALP to convert 4-digit Hex number into its equivalent BCD number and 5- digit

BCD number into its equivalent HEX number. Make your program user friendly to accept the choice

from user for: (a) HEX to BCD b) BCD to HEX (c) EXIT. Display proper strings to prompt the user

while accepting the input and displaying the result. (Wherever necessary, use 64-bit registers).

section .data

menu db 10,"_-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_ MENU _-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_-_",10

db 10,"1) HEX to BCD Conversion.",

db 10,"2) BCD to HEX Conversion.",

db 10,"3) Exit.",

db 10,"Enter Your Choice : "

menuLen equ $ - menu

hbMsg db 10,"HEX to BCD"

db 10,"Enter Maximum 4 Digits HEX Number : "

hbMsgLen equ $ - hbMsg

bmsg db "Equivalent BCD : "

bmsgLen equ $ - bmsg

bhMsg db 10,"BCD to HEX"

db 10,"Enter Maximum 5 Digits BCD Number : "

bhMsgLen equ $ - bhMsg

hmsg db "Equivalent HEX : "

hmsgLen equ $ - hmsg

newline db 10,13

newlineLen equ $ - newline

section .bss

buf resb 6

bufLen equ $ - buf

digitcount resb 1

ans resw 1

charAns resb 4

%macro print 2

mov rax, 1

mov rdi, 1

mov rsi, %1

mov rdx, %2

syscall

%endmacro

%macro read 2

mov rax, 0

mov rdi, 0

mov rsi, %1

mov rdx, %2

syscall

%endmacro

%macro exit 0

mov rax, 60

mov rdi, 0

syscall

%endmacro

section .text

global _start

_start:

menuLabel:

print menu, menuLen

read buf, bufLen

mov al, [buf]

case1:

cmp byte[buf],'1'

jne case2

call hexToBcd

print newline, newlineLen

jmp menuLabel

case2:

cmp byte[buf],'2'

jne case3

call bcdToHex

print newline, newlineLen

jmp menuLabel

case3:

cmp byte[buf],'3'

je exit

hexToBcd:

print hbMsg, hbMsgLen

call hexNum

mov rax, rbx

mov rbx, 10

back:

xor rdx, rdx

div rbx

push rdx

inc byte[digitcount]

cmp rax, 0h

jne back

print bmsg, bmsgLen

print_bcd:

pop rdx

add dl, 30h

mov [charAns], dl

print charAns, 1

dec byte[digitcount]

jnz print_bcd

ret

hexNum:

read buf, bufLen

mov rbx, 00

dec rax

mov rcx, rax

mov esi, buf

mov rax, 00

l3:

shl rbx, 4

mov al, [esi]

cmp al, 39h

jbe sub30

sub al, 07h

sub30:

sub al, 30h

add rbx, rax

inc esi

loop l3

ret

bcdToHex:

print bhMsg, bhMsgLen

read buf, bufLen

mov esi, buf

dec rax

mov rcx, rax

xor rax, rax

mov rbx, 10

back1:

xor rdx, rdx

mul rbx

xor rdx, rdx

mov dl, [esi]

sub dl, 30h

add rax, rdx

inc esi

dec rcx

jnz back1

mov [ans], ax

print hmsg, hmsgLen

mov ax, [ans]

call disp16

ret

disp16:

mov esi, charAns+3

mov rcx, 4

cnt:

mov rdx, 0

mov rbx, 16

div rbx

cmp dl, 09h

jbe add30

add dl, 07h

add30:

add dl, 30h

mov [esi],dl

dec esi

dec rcx

jnz cnt

print charAns, 4

ret

Output: